Complete radio link budget from all earth stations:

| Frequency | 437.5 | MHz |  |
| --- | --- | --- | --- |
| Orbital Altitude | 500 | Km |  |
| Elevation Angle | 35 | Deg |  |
| Distance | 815.089 | Km |  |
|  | -10 | dBW |  |
| Spacecraft Antenna Gain | 2.2 | dBi |  |
| Spacecraft Line Loss | 1 | dB |  |
| EIRP | -8.8 |  |  |
|  | GFSK (2K00 F1D & 1K00 F1D) | CW Morse (500H A1A) |  |
| Free Space Path Loss:: | 143.4737 | 143.4737 | dB |
| Antenna Pointing Loss | 1 | 1 | dB |
| Antenna Polarization Loss: | 1 | 1 | dB |
| Atmospheric Loss: | 2 | 2 | dB |
| Ionospheric Loss: | 1 | 1 | dB |
| Rain Loss: | 0 | 0 | dB |
| Isotropic Signal Level at GS | -157.274 | -157.274 | dB |
| GS Antenna Pointing Loss | 0.2 | 0.2 | dB |
| GS Antenna Gain | 14 | 14 | dB |
| GS Transmission Loss | 2 | 2 | dB |
| GS Effective Noise Temperature | 635 | 635 | K |
| GS Figure of merit (G/T) | -16.2277 | -16.2277 | dB/K |
| Signal Power at Input of LNA | -145.474 | -145.474 | dBW |
| Boltzman’s constant | -228.6 | -228.6 | dbW/K/Hz |
| Receiver Bandwidth | 2000 | 500 | Hz |
| Receiver Noise Power | -167.562 | -173.583 | dBW |
| SNR at Receiver | 22.0883 | 28.1089 | dB |
| Requested SNR | 12.8 | 10 | dB |
| Link Margin | 9.288301 | 18.1089 | dB |