| Frequency: | 437.5 |  | MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Orbital Altitude | 500 |  | km |  |  |  |
| Elevation Angle | 35 |  | Deg |  |  |  |
| Distance: | 815.089 |  | km |  |  |  |
|  | -10 |  | dBW |  |  |  |
| Spacecraft Antenna Gain | 2.2 |  | dBi |  |  |  |
| Spacecraft Line Loss | 1 |  | dB |  |  |  |
| EIRP | -8.8 |  |  |  |  |  |
|  | GFSK(2K00F1D) | CW Morse (500H A1A) |  | RTTY FSK (515H F1B) |  |  |
| Free Space Path Loss: | 143.4737 |  | 143.4737 | 143.4737 |  | dB |
| Antenna Pointing Loss: | 1 |  | 1 | 1 |  | dB |
| Antenna Polarization Loss: | 1 |  | 1 | 1 |  | dB |
| Atmospheric Loss: | 2 |  | 2 | 2 |  | dB |
| Ionospheric Loss: | 1 |  | 1 | 1 |  | dB |
| Rain Loss: | 0 |  | 0 | 0 |  | dB |
| Isotropic Signal Level at GS | -157.274 |  | -157.274 | -157.274 |  | dB |
| GS Antenna Pointing Loss | 0.2 |  | 0.2 | 0.2 |  | dB |
| GS Antenna Gain | 14 |  | 14 | 14 |  | dB |
| GS Transmission Loss | 2 |  | 2 | 2 |  | dB |
| Ground Station Effective Noise Temperature: | 635 |  | 635 | 635 |  | K |
| Ground Station Figure of Merrit (G/T): | -16.2277 |  | -16.2277 | -16.2277 |  | dB/K |
| Signal Power at Input of LNA | -145.474 |  | -145.474 | -145.474 |  | dBW |
| Boltzman's Constant | -228.6 |  | -228.6 | -228.6 |  | dbW/K/Hz |
| Receiver Bandwidth | 2000 |  | 500 | 515 |  | Hz |
| Receiver Noise Power | -167.562 |  | -173.583 | -173.454 |  | dBW |
| SNR at Receiver | 22.0883 |  | 28.1089 | 27.98053 |  | dB |
| Required SNR | 12.8 |  | 10 | 10 |  | dB |
| Link Margin | 9.288301 |  | 18.1089 | 17.9805 |  | dB |